The wind on your side

Amazone WindControl is now available independent of ArgusTwin.

The wind is always blowing somewhere in the world and this represents a major challenge in maintaining an even fertiliser spread pattern. The Amazone WindControl system means that the effect of the wind on the spread pattern is permanently monitored and automatically compensated for. WindControl can now also be used for the ZA-TS mounted spreaders and ZG-TS trailed spreaders independently of the ArgusTwin spread fan monitoring system.

The conflict with wind when mineral fertilising

Winds can be strong or light, blow from different directions or blow at a constant speed one moment and in gusts the next. As a result, the effect of the wind on the spread pattern never stays the same. The wind speed and the wind direction vary both within the tramline and in comparison to the adjacent tramline.

In fertiliser spreading, wind is a parameter which the user cannot influence directly but must always keep in mind. As a rule, the main period of use for a centrifugal broadcaster is during the main growing season in spring. At this time of year, the site-related probability of having to work in windy conditions is particularly high when viewed over the year as a whole.

Headwinds or tailwinds stretch or compress the distribution pattern. The lateral distribution is not affected in either case, so that the throwing distance and therefore the effective working width remain unchanged. On the other hand, crosswinds change the lateral distribution significantly. The distribution pattern is compressed on one side and stretched on the other, resulting in an asymmetrical spread pattern. So the task of WindControl is to permanently monitor the spread pattern and adjust the fertiliser spreader settings in such a way that a symmetrical spread pattern is restored.

Windless conditions "at the touch of a button" widens the application window

A high-frequency wind sensor, mounted on the fertiliser spreader, records both the wind speed and the wind direction and transmits this information to the job computer. This processes the data with the forward speed and calculates new settings for both the delivery system and the spreading disc speed, which are then automatically adjusted. If there is a crosswind, the disc speed on the side facing the wind is increased and the delivery system rotated outwards. At the same time, the speed of the downwind side is reduced and the delivery system rotated inwards. This automatically counteracts the wind effect and ensures an accurate distribution. In addition, the use of WindControl results in larger time windows for fertilisation.

The wind at a glance

In addition to all the important parameters for fertiliser spreading, the operator always has an overview of the current wind situation such as the wind direction, wind strength and gustiness in the work menu. Traffic light colours are used to signal the extent to which WindControl is still able to compensate for the effect of the wind. The system also issues a warning message if the limit to the amount of control is reached when the wind is too strong or if the gusting is changing too frequently.

The mast holding the wind sensor is automatically extended when the spreading discs are actuated. In doing so, the sensor protrudes above the tractor cab so as not to measure any turbulence generated by the tractor. When the spreading discs are switched off, the wind sensor retracts to the protected position between the tractor and the spreader. Manual operation is also possible.

Perfect results under any circumstances

Amazone offers the ArgusTwin spread fan monitoring system for the ZA-TS and ZG-TS centrifugal broadcasters in addition to WindControl. ArgusTwin enables the spread pattern from both the right and left hand disc to be monitored constantly by 14 radar sensors. If there are any deviations, the lateral distribution is automatically optimised by an adjustment via the electric delivery system. This can be carried out individually for each side, thereby ensuring optimum lateral distribution. ArgusTwin ensures a constant spread pattern even when spreading on slopes, using non-homogeneous spreading materials, or when moisture causes changes in the spreading process.

For further information click here

AMAZONE WIndControl ensures an optimum lateral distribution even in crosswinds.

The settings for the fertiliser spreader are permanently recalculated and automatically adjusted via the high-frequency wind sensor.

In addition to all the important parameters for fertiliser spreading, the operator always has an overview of the current wind situation in the work menu.

Without WindControl: Crosswinds affect the distribution pattern and change the lateral distribution.

With WindControl: WindControl counteracts crosswinds ensuring an optimum spread pattern at all times.

About AMAZONE

AMAZONEN-WERKE H. Dreyer SE & Co. KG is based in Hasbergen-Gaste in Germany and manufactures agricultural and groundcare machinery. The owner-managed company employs around 1900 people at nine different production sites in Germany, France, Russia and Hungary. The agricultural machinery range includes soil tillage implements, seed drills, fertiliser spreaders and plant protection equipment. SCHMOTZER Hacktechnik has been part of the AMAZONE Group since 2019. Based on these core competencies, AMAZONE is now the specialist for intelligent crop production in agriculture. Further information: www.amazone.net