Mounted combination: Precea precision air seeder with CombiDisc 3000 compact disc harrow

The best of both worlds

The perfect combination solution for light to medium soils at forward speeds of up to 15 km/h

The Precea high-speed precision air seeder ensures very precise seed placement as a result of its new over-pressure separator. The seed is separated individually into each seed row and is introduced into the soil via the propulsion channel. The Precea 3000-A Super is suitable for forward speeds up to 15 km/h and is thus ideal for combination with the CombiDisc 3000 mounted compact disc harrow.

Seedbed preparation and sowing row crops in a single pass

The CombiDisc mounted compact disc harrow serves as the passive soil tillage for a wide range of AMAZONE drill combinations. This allows AMAZONE also to offer a passive soil tillage machine, alongside its PTO-driven soil tillage ranges, such as the rotary harrow and the rotary cultivator for its drill combinations. The combination with the Precea-A precision harrow-mounted seed drill is new.

Ideal for seedbed preparation

The CombiDisc has a working width of 3 m and is comprised of two rows of discs, one after the other. Each row of discs is comprised of 12 serrated discs, each having a diameter of 410 mm. In each case, two discs are coupled on a single disc arm. The disc arms are carried in rubber spring elements as standard. The disc bearings are maintenance-free, of course.

Working depths of 3 to 8 cm can be achieved. This makes the disc harrow ideal for seedbed preparation on light to medium soil structures. The working depth of the two rows of discs can be adjusted mechanically, or alternatively via a hydraulic cylinder from the tractor cab.

The rapidly-rotating discs provide a very good crumb structure. The genuine working width of 3 m ensures full-area soil tillage in front of the seed drill. Perfect levelling of the seedbed is ensured by adjustable disc carriers in the wheel tracks and at the machine ends.

Proper reconsolidation of the soil is provided by a following roller from the wide range of AMAZONE rollers.

The seed drill is quickly and safely attached via the QuickLink quick coupling. The CombiDisc 3000 compact disc harrow is coupled to the tractor via Cat. 2 or Cat. 3 lower link mountings. Simple mounting is supported by the incorporation of a hose rail to facilitate connection of the hydraulic hoses.

Precea 3000-ACC Super with CombiDisc 3000

Coupling of the mounted CombiDisc 3000 compact disc harrow to the Precea precision air seeder via QuickLink

Precea 3000-ACC mounted on the CombiDisc 3000

About AMAZONE

AMAZONEN-WERKE H. Dreyer SE & Co. KG is based in Hasbergen-Gaste in Germany and manufactures agricultural and groundcare machinery. The owner-managed company employs around 1900 people at nine different production sites in Germany, France, Russia and Hungary. The agricultural machinery range includes soil tillage implements, seed drills, fertiliser spreaders and plant protection equipment. SCHMOTZER Hacktechnik has been part of the AMAZONE Group since 2019. Based on these core competencies, AMAZONE is now the specialist for intelligent crop production in agriculture. Further information: www.amazone.net