New AMAZONE KE 02 Rotamix rotary harrow – for seed drill combinations

AMAZONE has extended its new KE 02 Rotamix rotary harrow range with two new models. The new models, the KE 02-150 in working widths of 2.5 m and 3 m for tractors up to 150 hp, and the KE 02-190 in working widths 3 m, 3.5 m and 4 m for tractors up to 190 hp demonstrate their strengths, particularly with their intensive soil crumbling. The range is topped off with the KE 02-240 for the more powerful horsepower classes up to 240 hp.

The Rotamix system – close-coupled, compact and two extra tool carriers at 3 m

The KE 02 rotary harrow, with the Rotamix system, has four tine carriers per metre of working width. The incoming power is evenly distributed across these rotors. The trailing tines produce a very good crumb structure, even on heavy soils. The new rotary harrows are therefore very well suited for seedbed preparation, particularly following the plough.

The number of tine carriers enables the use of a smaller spur gear diameter. This means that the sturdy gear trough can be designed to be very compact and therefore lightweight. The short design ensures that an attached seed drill can be positioned very close to the tractor. This reduces the leverage on the tractor and so the lifting power requirement is also less than with other rotary harrows.

The tried and tested Quick+Safe system provides the 290 mm tines with an integrated stone release and the tines can be easily exchanged without the need for tools.

A higher class of performance with the DirectDrive gearbox

The Long-Life-Drive system, including the DirectDrive gearbox, ensures a long service life and maximum smooth running. The double sealing system with cassette seal and labyrinth seal protects against dust, moisture and dirt ingress.

The heart of the new rotary harrows is the DirectDrive gearbox, via which the higher power flow is transmitted directly to the spur gears of the tine carriers. On the new KE 02 Rotamix there is no power redirection thus ensuring an excellent, low-wear, power transmission. The new rotary harrows can be used with PTO speeds of 540, 750 or 1,000 rpm. Exchange gear sets allow the tine carrier speed to be adapted to different soil conditions and to the PTO speed of the tractor.

Thanks to the new drive train with the high-efficiency DirectDrive gearbox, the KE 2502-150 and KE 3002-150 are approved for tractor outputs up to 150 hp. By using the drive train with more hardened and tempered components, the KE 3002-190, KE 3502-190 and KE 4002-190 are approved for use up to a tractor output of 190 hp. The KE 3002-240 and KE 4002-240 rotary harrows are even approved for up to 240 hp thanks to their even stronger DirectDrive gearboxes.

Setting and tools

Using the optional hydraulic depth control, the rotary harrows can easily be adapted to the actual conditions in the field from the tractor cab on the move. A graduated scale ensures an easy monitoring of the working depth. As an alternative, the working depth can be set using an eccentric pin in a series of holes.

The KE 02-150 and 190 models have a rigid, Cat. 3N lower link cross shaft.

The levelling board for levelling the soil is guided precisely at the correct height by the packer roller and is able to be deflected upwards using the integrated overload safety protection. A universal setting tool is supplied for fine-tuning the settings, which can be used for many tasks, such as setting the height adjustment of the side plates or for setting the track markers.

Versatile roller range

Numerous roller types with different diameters for targeted reconsolidation are available for a wide range of site-specific requirements. The cage roller, as lightweight entry-level packer for controlling the depth of the rotary harrow, gives full-area but only slight reconsolidation, whereas AMAZONE offers the tooth packer roller for full-area crumbling and reconsolidation. The wedge ring roller, for light to medium soil types, reconsolidates in strips in the seed row. Optionally also with Matrix profile for an even better drive in lighter soil types. The roller programme is rounded off with the trapezium roller, also giving strip-wise reconsolidation in the seed row.

KE 3002-190 Rotamix with Centaya 3000 Special harrow-mounted seed drill

Video of the KE 3002-190 Rotamix in use: 
www.amazone.net/yt-KE-3002-190

Video of the KE 3002-240 Rotamix in use: 
www.amazone.net/yt-KE-3002-240-Rotamix

KE 3002-190 Rotamix with four tine carriers per meter of working width

The side plate can be opened for drilling by means of the universal tool

KE 3002-190 Rotamix with DirectDrive gearbox

KE 3002-190 Rotamix with TRW 500 trapezium ring roller

The rotary harrow can be equipped with marker boards and road lights for solo operation

Illustrations, content and technical data are not binding and may differ depending on the level of equipment. Country-specific road traffic regulations apply and must be complied with, meaning that special approval may be required. The permissible axle loads and total weights of the tractor have to be checked. Not all the listed combination options are possible with all tractor manufacturers.

About AMAZONE

AMAZONEN-WERKE H. DREYER SE & Co. KG is based in Hasbergen-Gaste in Germany and manufactures agricultural and groundcare machinery. The owner-managed company employs around 2000 people at nine different production sites. The agricultural machinery range includes soil tillage implements, seed drills, fertiliser spreaders and crop protection equipment. SCHMOTZER Hacktechnik has been part of the AMAZONE Group since 2019. Based on these core competencies, AMAZONE is now the specialist for intelligent crop production in agriculture. Further information: www.amazone.net