AMAZONE innovation at demopark – InsectGuard

Amazone is presenting its complete groundcare machinery range for green space maintenance and winter road gritting at this year's demopark in Eisenach (Germany). Amongst other things, the team of experts on the Amazone Stand D-418 will demonstrate the Profihopper SmartLine self-propelled mower/collector in practice. With its SmartCut exact cut rotor and the high-performance PowerCompactor collection and auger conveyor system, the Profihopper can be used for mowing, scarifying and leaf collection throughout the year. The new comfort cab ensures comfortable working even on long days and offers the driver protection in all weather conditions.

NEW at demopark: Protect nature with InsectGuard
The InsectGuard insect protection system is now available for the Profihopper. The system is installed out the front of the cutting deck and reduces the number of insects killed when mowing meadows and grass verges. Extensively managed meadows and communal green spaces in parks and leisure facilities also provide a habitat for a wide range of insects which need to be protected. The InsectGuard is a tine carrier which ensures that the grass in front of the cutting deck is moved vigorously. This causes insects to fly away, thereby preventing them from being mown over by the Profihopper.

The ingenious kinematic mounting system and the tines provide active impact protection. In the event of a collision with an obstruction, the tines deform and the unit moves backwards. The kinematic mounting system returns the tine carrier to its working position during the subsequent reversing. The tine carrier is simply folded upwards for transport or when mowing without the need for insect protection. The angle of attack of the tines can be varied in the working position. The unique kinematic mounting system also allows work to be carried out close to trees and bushes.

NEW: Amazone now offers the InsectGuard insect protection system for the Profihopper self-propelled mower/collector.

Reliable operation all year round

Furthermore, the Groundkeeper mounted mower/collector will also be on show. These models are also equipped with the SmartCut exact cut rotor with a V-shaped blade arrangement for the optimum cutting and collecting quality, even on wet grass. The Groundkeeper is available with ground-level discharge or high-lift discharge in a variety of working widths and impresses with its comfortable operation. In the HorseHopper variant, the machine can also be used in paddocks for optimising pasture hygiene.

More precision in winter road gritting

The range on show is rounded off by the mounted spreaders for winter road gritting. These include the IceTiger with its innovative Smart Precision System which enables the economical and environmentally-friendly application of a wide variety of spreading materials. Even extremely small quantities are precisely applied via the hydraulically-driven floor belt and the precision spreading disc. The electric delivery point adjustment via ISOBUS provides maximum comfort. The IceTiger can be equipped with a brine device for the application of pre-wetted salt.

Illustrations, content and technical data are not binding and may differ depending on the level of equipment. Country-specific road traffic regulations apply and must be complied with, meaning that special approval may be required. The permissible axle loads and total weights of the tractor should be checked. Not all the listed combination options are possible with all tractor manufacturers.

About AMAZONE

AMAZONEN-WERKE H. DREYER SE & Co. KG is based in Hasbergen-Gaste in Germany and manufactures agricultural and groundcare machinery. The owner-managed company employs around 2000 people at nine different production sites. The agricultural machinery range includes soil tillage implements, seed drills, fertiliser spreaders and crop protection equipment. SCHMOTZER Hacktechnik has been part of the AMAZONE Group since 2019. Based on these core competencies, AMAZONE is now the specialist for intelligent crop production in agriculture.

In addition to this, Amazone offers groundcare machinery for park and green space maintenance as well as winter road gritting. Further information: www.amazone.net