New double harrow for the Cobra shallow tine cultivator

Weed control with the Cobra shallow tine cultivator in combination with the double harrow

The 6-stagger Cobra shallow tine cultivator is characterised by its flexibility in respect to stubble cultivation and can be used for that slightly deeper second and third tillage pass as well as for catch crop destruction and seedbed preparation.

Mechanical weed control during that second pass is becoming more and more important for field hygiene as a result of increasing resistance and the reduced availability of plant protection agents.

Double harrow

In addition to a choice of 9 AMAZONE following rollers as double or single units, a double harrow is now available as a follow-up tool for reconsolidating the soil.

The double harrow really comes into its own in mechanical weed control during the second or third pass. The weeds or volunteer grains cut down by the duckfoot shares are deposited on the soil surface by the double harrow. Furthermore, the vibration of the harrow tines separates the soil from the roots of the weeds and volunteer grains. The harrow pulls the roots out of the soil. Separating the soil from the roots and then depositing them on the surface means that the weeds and volunteer grains dry out in the sun.

In addition, the double harrow facilitates levelling and improved distribution of the straw on the field. The soil can also warm up and dry more quickly in the spring owing to the reduced level of reconsolidation.

When working with the double harrow, the depth of the machine is controlled via the 4 integrated support wheels at the front and via the TX transport running gear at the rear. The hydraulic connection of the support wheels to the running gear also enables the working depth to be infinitely adjusted whilst the machine is in work when using the double harrow. A tine is mounted behind each of the transport wheels in order to loosen any compaction caused by the running gear.

- Mechanical weed control thanks to full-surface cut and trailing double harrow

- Improvement in field hygiene and reduction in the use of plant protection agents

- Supplementary straw distribution and soil levelling

- Faster drying and warming of the soil in the spring

The AMAZONE Cobra with the double harrow making a second pass for the control of volunteer grains and weeds

The double harrow provides improved lateral distribution of the straw after combining

Thanks to the double harrow, the soil is separated from the volunteer grains and the bare plants deposited on the surface. There the plants dry off in the sun

Illustrations, content and technical data are not binding and may differ depending on the level of equipment. Country-specific road traffic regulations apply and must be complied with, meaning that special approval may be required. The permissible axle loads and total weights of the tractor should be checked. Not all the listed combination options are possible with all tractor manufacturers.

About AMAZONE

AMAZONEN-WERKE H. DREYER SE & Co. KG is based in Hasbergen-Gaste in Germany and manufactures agricultural and groundcare machinery. The owner-managed company employs more than 2000 people at nine different production sites. The agricultural machinery range includes soil tillage implements, seed drills, fertiliser spreaders and crop protection equipment. SCHMOTZER Hacktechnik has been part of the AMAZONE Group since 2019. Based on these core competencies, AMAZONE is now the specialist for intelligent crop production in agriculture. Further information: www.amazone.de