New FW 580 flex ring roller for mounted soil tillage implements

AMAZONE introduces the FW 580 flex ring roller, a new packer roller for the Cenio and Cenius mounted cultivators and Catros mounted compact disc harrows. Thanks to its open design with a ring spacing of 200 mm and its narrow roller core, the roller, with a total diameter of 580 mm, is suitable for a broad range of applications. Its performance is impressive on both sandy and loamy soils.

Thanks to the continuous spring action of the rings, the FW 580 produces a lot of fine soil whilst ensuring deep reconsolidation and a loose soil surface. These properties make it perfect for seedbed preparation. In addition, the solid rings reduce the carry-over of soil and weed seeds. This helps to improve field hygiene, especially when working in different fields.

The excellent cleaning effect created by the spring action of the rings is also very advantageous. This ensures reliable operation, even in damp and sticky soil conditions. Thanks to the small roller tube diameter of 140 mm, the roller offers excellent through-passage on light sandy soils. The arrangement of the rings ensures a reliable self-driving effect.

The optional spring-loaded cleaning system offers additional functionality by keeping the gaps between the individual rings clear. This effectively prevents the roller from blocking up, even under the most demanding soil conditions. The harrow system, also an option, ensures finer crumbling of the seedbed and supports precise placement of the seed. Both systems also give additional levelling on very light soils.

Thanks to its low own weight, the flex ring roller can be easily lifted by smaller tractors both at the headland and when travelling on the road.

- Good turning properties, thanks to a diameter of 580 mm

- High load-bearing capacity, thanks to robust flex rings

- Suitable for varying soil types – from sandy to loamy

- Deep reconsolidation yet with a loose soil surface

- Very good cleaning and crumbling effect, thanks to the spring action of the rings

- Additional crumbling and levelling with optional sprung cleaning system and harrow system

- Low weight for easy lifting at the headland

- Cost-effective and reliable

The FW 580 flex ring roller offers a reliable solution with high level of operational reliability and flexibility

Flex ring roller FW 580

Flex ring roller FW 580 with sprung cleaning system

Illustrations, content and technical data are not binding and may differ depending on the level of equipment. Country-specific road traffic regulations apply and must be complied with, meaning that special approval may be required.

About AMAZONE

AMAZONEN-WERKE H. DREYER SE & Co. KG is based in Hasbergen-Gaste in Germany and manufactures agricultural and groundcare machinery. The owner-managed company employs more than 2500 people at nine different production sites. The agricultural machinery portfolio includes soil tillage implements, seed drills, fertiliser spreaders, plant protection equipment and hoes. Based on these core competencies, AMAZONE is now the specialist for intelligent crop production in agriculture. In addition to this, with its groundcare range, AMAZONE offers versatile machinery for park and green space maintenance as well as winter road gritting.

Further information: https://amazone.net