New hydraulic overload protection

Higher operational performance with the new C-Mix Ultra tine system for Cenius trailed cultivators and the Ceus disc & tine combination cultivators

Amazone has extended the build options on its Cenius-2TX trailed cultivator models, in working widths of 4 to 8 m, and the Ceus-2TX trailed disc & tine combination cultivators, which are available in working widths from 4 to 7 m, with the addition of the new C-Mix Ultra tine. The new tine system is equipped with an automatic overload protection that utilises hydraulic cylinders and so offers a high reliability and durability, especially in extreme operating conditions.

The strengths of the C-Mix Ultra tine becomes apparent in challenging soil conditions where the tine protection system is being regularly overloaded. This new hydraulic variant enables the trigger force to be adapted to the conditions and is infinitely-adjustable up to 800 kg. This provides a high level of operational reliability, because the machine optimally maintains the desired working depth of up to 30 cm even under the toughest of conditions thanks to the high trip clearance of the individual tines. In addition, the Ultra overload protection system protects the cultivator in a unique way: The system, consisting of the hydraulic cylinders installed on the tines and the centralised nitrogen accumulators, provides highly damped resetting of the C-Mix Ultra tine after triggering. This also has a very beneficial effect on the wear and tear of the overload protection system, even in the case of frequent triggering.

The number of nitrogen accumulators is adjusted to the respective number of tines so that there is only a small pressure difference when several overload protection devices are triggered simultaneously.

Stepless adjustment of the system is easily and conveniently carried out from the front on the drawbar. The pressure gauge is also located there and can be easily viewed from the tractor cab for convenient monitoring.

The new C-Mix Ultra tine can be combined with the complete C-Mix share system. The extensive range offers the right share for every application. Some of the shares are available as highly wear-resistant HD variants.

The C-Mix Clip quick-change system, which enables the points to be exchanged easily and conveniently, is also offered. The point is separated from the guide plate in next to no time. This ensures short set-up times and, above all, a reduction in the wear costs.

Video Ceus 5000-2TX:  www.amazone.de/ceusultra

The C-Mix Ultra tine is damped via the hydraulic cylinder when it is reset after triggering.

The high trigger force of up to 800 kg ensures a consistent working depth and protects the frame.

About AMAZONE

AMAZONEN-WERKE H. DREYER SE & Co. KG, based in Hasbergen-Gaste in Germany, manufactures agricultural and groundcare machinery. The owner-managed company employs around 1900 people at nine different production sites in Germany, France, Russia and Hungary. The agricultural machinery range includes soil tillage implements, seed drills, fertiliser spreaders and plant protection equipment. Schmotzer Hacktechnik has been part of the AMAZONE Group since 2019. Based on these core competencies, AMAZONE is now the specialist for intelligent crop production in agriculture.

Further information: www.amazone.net