Hydraulic tramline offset for Precea precision air seeder

Maintaining the seed rate even when tramlining

Hydraulic tramline offset

The idea of the hydraulic tramline offset is to produce crop care tramlines without having to switch off the singling units, thus avoiding reducing the seed rate. The singling units are not switched off for the tramline, they are simply moved apart. On the AMAZONE Precea, the fertiliser coulter is also moved alongside the displacement of the sowing units. The maximum offset distance of the units is 400 mm. However, this distance can be limited if the full distance is not required.

The hydraulic tramline offset can be either symmetric or asymmetric. With a symmetric offset, the cylinders for offset on the right-hand side and on the left-hand side of the machine are powered at the same time. With an asymmetric offset, just one cylinder is powered and thus just one unit is displaced. For example, the driver produces a tramline with just one wheel mark for the following crop care machine in the first pass, and then produces the second wheel mark in a second pass. The system is incorporated in the ISOBUS machine software to make it easier for the driver, and it reacts automatically as soon as comes to the generation of the tramline.

The hydraulic tramline offset system is very flexible and thus offers many combination options for track width, tyre width and sprayer boom width, depending on the working width of the precision air seeder. The hydraulic tramline offset can be used on all Precea models, with the exception of the telescopic machines.

The advantages at a glance:

- Optimum yield potential since the seed rows do not need to be switched off, they are just displaced

- Relief for the driver by automatic detection and adaption to make the tramline

- Lack of plant damage during subsequent working in the crop, as a result of the existing tramline

- Optimum fertiliser supply since the fertiliser coulter and the singling unit are displaced together

Hydraulic tramline offset on the Precea 6000-2FCC – asymmetric arrangement

The asymmetric offset permits track widths up to 2.1 m

Symmetric tramline offset on the Precea 6000-2

Hydraulic tramline offset on the Precea 6000-2FCC – symmetric arrangement

About AMAZONE

AMAZONEN-WERKE H. Dreyer SE & Co. KG is based in Hasbergen-Gaste in Germany and manufactures agricultural and groundcare machinery. The owner-managed company employs around 1900 people at nine different production sites in Germany, France, Russia and Hungary. The agricultural machinery range includes soil tillage implements, seed drills, fertiliser spreaders and plant protection equipment. SCHMOTZER Hacktechnik has been part of the AMAZONE Group since 2019. Based on these core competencies, AMAZONE is now the specialist for intelligent crop production in agriculture. Further information: www.amazone.net