Centaya-C harrow-mounted seed drill for combined sowing

AMAZONE is extending the range of pneumatic harrow-mounted seed drill with the new Centaya-C. The special feature of the sowing combination is the new 2,000 l twin chamber hopper. This makes it possible to apply several materials in a single pass. The new Centaya-C Super is available in working widths of 3, 3.5 and 4 m.

Wide range of applications

As a result of the increasing challenges in professional crop establishment, the requirements for a modern seeding system as the basis for achieving the highest yields are constantly increasing. Precise seed metering and exact depth placement therefore have to have top priority. Furthermore, there is a particular focus on the simultaneous application of more than one seed type, or the parallel application of fertiliser with the seed. The twin-chamber hopper means that the new Centaya-C Super now offers the option of sowing companion crops and undersowing in addition to the main cash crop. These are of great benefit for weed suppression and for increasing erosion control and biodiversity. In addition, the simultaneous application of fertiliser is an efficient solution that leads to rapid seedling development and high field emergence. This sowing technique offers farmers and contractors a variety of agronomical methods and a high degree of flexibility by the combination of seed and fertiliser. Moreover, fuel and time can be saved through a reduction in the number of passes.

Up to three different placement points

The Centaya-C Super can be equipped with either the RoTeC pro single disc or the TwinTeC double disc coulters.

The 400 mm RoTeC pro coulter enables the application of two media utilising the single-shoot process. In this case, the individually-metered materials are brought together in the single disc coulter and fed to a single entry point using the same conveying system. This enables, for example, a small amount of fertiliser to be placed directly in the row along with the grain.

In conjunction with the TwinTeC double disc coulter, it is even possible to apply two different seed types or seed and fertiliser at two different entry points utilising the double-shoot process. In this case, the first medium is embedded in the soil via the TwinTeC coulter and the second medium is conveyed to an additional outlet on the TwinTeC coulter via a separate conveyor system and placed in the soil in front of the depth guidance roller. This offset placement means, for instance, that fertiliser can be used precisely, thereby ensuring a more comprehensive supply to the plant.

The Centaya-C Super can also be used in conjunction with the GreenDrill 200 catch crop seeder box for the simultaneous sowing of catch crops or fine seeds. This combination allows a third material to be applied to the soil surface via baffle plates. In this respect, the seed is fed directly from the 200 l mounted hopper down to baffle plates behind the seed drill.

Precise, smart and highly productive

The ISOBUS-controlled Centaya-C Super enables separate and precise metering of the various application materials via the infinitely adjustable electric metering units with sowing rates of 0.5 - 400 kg/ha. The Centaya-C Super can be used at forward speeds of up to 12 km/h with row spacings of 12.5 and 15 cm.

Filling is easy thanks to the large hopper opening. The seed hopper and the individual metering units can be easily accessed via the loading board. The SmartCenter permits user-friendly and precise calibration and machine setting. The application rates and roller speed are clearly displayed on the AMAZONE ISOBUS terminal AmaTron 4. The driver can change the seed rate from the tractor seat during work without any problems.

The drill can be equipped with a quick-emptying device for fast seed changeover between jobs. An optional storage rack is available for storing additional sacks of seed.

The 2,000 l twin-chamber hopper means that the Centaya-C Super is highly productive. The hopper can be divided individually in a ratio of either 60:40 or 70:30 depending on the customer’s requirements. The special profile and the positioning of the metal hopper close to the tractor ensure a low lifting power requirement.

QuickLink quick coupling system

The Centaya-C Super harrow-mounted seed drill can be very easily and quickly connected without tools to the various AMAZONE soil tillage implements via the QuickLink quick coupling system. Depending on the soil conditions, it can be combined with the KE 02 rotary harrow or the KG and KX rotary cultivators. On very light soils, the Centaya-C Super can be used with the compact disc harrow CombiDisc.

Centaya-C with second distributor head for the simultaneous metering of a second material

The second material is directed to an additional entry point on the TwinTeC coulter via a separate conveying section and is placed before the depth guidance roller

The high-capacity electric metering units are infinitely variable

Several materials can be applied simultaneously with the new AMAZONE Centaya-C Super using the 2,000 l twin-chamber hopper.

Illustrations, content and technical data are not binding and may differ depending on the level of equipment. Country-specific road traffic regulations apply and must be complied with, meaning that special approval may be required. The permissible axle loads and total weights of the tractor have to be checked. Not all the listed combination options are possible with all tractor manufacturers.

About AMAZONE

AMAZONEN-WERKE H. DREYER SE & Co. KG is based in Hasbergen-Gaste in Germany and manufactures agricultural and groundcare machinery. The owner-managed company employs around 2000 people at nine different production sites. The agricultural machinery range includes soil tillage implements, seed drills, fertiliser spreaders and crop protection equipment. SCHMOTZER Hacktechnik has been part of the AMAZONE Group since 2019. Based on these core competencies, AMAZONE is now the specialist for intelligent crop production in agriculture. Further information: www.amazone.net