CurveControl for AMAZONE centrifugal broadcasters

Precise fertiliser application even around curves

CurveControl adjusts the lateral distribution to the various cornering speeds and corrects the movement of the spread pattern in centrifugal fertiliser broadcasters when driving around bends. This results in more uniformcrops and helps prevent of undesired over-fertilisation inside the bend as well as fertiliser placement beyond the field boundary.

The challenge: complex causal relationships

Curve compensation has already been implemented by AMAZONE in crop protection sprayers. However, the sprayer boom is located tight behind the physical pivot point of the machine. The application rate only has to be adjusted within the boom width for curve compensation.

The throwing distance of a centrifugal broadcaster is up to twice the working width and the application area is many metres behind the spreader depending on the type of fertiliser. Furthermore, the spread fan is kidney-shaped.

All this demands detailed knowledge of the way that the spreading unit works with the various fertilisers along with a corresponding complex implementation of a control algorithm. CurveControl therefore requires an exceptionally high degree of innovation and complexity for a fertiliser spreader.

Consequence: incorrect application caused by the turn

The current state of the art of modern centrifugal broadcasters has the disadvantage that over- and under-fertilised areas inside of the working width occur when negotiating bends as a result of different speeds. This effect is amplified by the combination of forward speed and swivelling of the spread pattern caused by the steering angle. This means that significant over-fertilisation arises in some areas as a result of these multiple overlaps, whereas gaps occur in other areas.

In addition to this, the swivelling of the spread fan at the edge of the field causes fertiliser to be applied beyond the field boundary. This must be avoided without fail with regard to resource conservation and to abide to the applicable regulations.

Perfect fertiliser distribution around curves

CurveControl prevents under- and over-fertilisation, which is undesirable from an ecological and economic point of view, and thereby minimises the risk of nutrients leaching into the ground water. Furthermore, incorrect application beyond the field boundary when negotiating bends is prevented by CurveControl. In addition to this, CurveControl can further improve the environmentally-friendly use of mineral fertiliser in combination with existing solutions such as ArgusTwin, WindControl and GPS part-width section control.

In summary, CurveControl closes an important, frequently underestimated gap and provides even distribution in all areas of the field. It will not only make a key contribution to environmental protection but will also improve the cost benefit result for every application.

- Environmentally-friendly 
No incorrect application when negotiating bends

- Resource-conserving 
Optimum use of the available fertiliser

- Economic 
Uniform crops in every field situation

Every field has borders and curves

Effect on fertiliser distribution when negotiating bends

- Incorrect application beyond the field boundary

- Over-fertilisation as a result of multiple overlaps

- Under-fertilisation caused by the move in the spread pattern

GPS view of a curve without CurveControl. The spread fan is located many metres behind the tractor and undesired swivel movements occur.

Perfect fertiliser distribution through CurveControl

- No incorrect application beyond the field boundary

- Even fertiliser distribution in all field areas

Illustrations, content and technical data are not binding and may differ depending on the level of equipment. Country-specific road traffic regulations apply and must be complied with, meaning that special approval may be required. The permissible axle loads and total weights of the tractor should be checked. Not all the listed combination options are possible with all tractor manufacturers.

About AMAZONE

AMAZONEN-WERKE H. DREYER SE & Co. KG is based in Hasbergen-Gaste in Germany and manufactures agricultural and groundcare machinery. The owner-managed company employs more than 2000 people at nine different production sites. The agricultural machinery range includes soil tillage implements, seed drills, fertiliser spreaders and crop protection equipment. SCHMOTZER Hacktechnik has been part of the AMAZONE Group since 2019. Based on these core competencies, AMAZONE is now the specialist for intelligent crop production in agriculture. Further information: www.amazone.de