ContourControl boom guidance for UF 02 mounted sprayers

Maximum precision and output for mounted sprayers

As an alternative to DistanceControl automatic boom guidance, AMAZONE now also offers the automatic active boom guidance system, ContourControl, for the UF 1602 and 2002 on boom widths from 27 m.

The ContourControl active boom guidance guarantees precise application at high forward speeds for maximum work rates and yet exact, close distances to the target surface and so resulting in less drift. This is an important component in precise, efficient and environmentally-friendly crop protection, even on small to medium-sized farms.

ContourControl

The AMAZONE ContourControl active boom guidance is a ground-breaking fully automatic hydraulic boom guidance system which now can also be used on mounted sprayers. The boom tilt is controlled by a hydraulic cylinder which is pre-pressurised on both sides. Rapidly switching hydraulic valves bring the boom to the desired position. This actively reduces any of the various effects caused by uneven ground and other influences while driving. The cylinders for positive and negative angling of the two outer ends of the boom are controlled using the same principle. This allows the boom to follow highly uneven topography to maintain an optimal distance to the target surface across the entire working width, even on extremely hilly terrain.

Less than 50 cm distance to the target surface with 25 cm nozzle spacing

Distances of less than 50 cm to the target surface can also be precisely maintained using the AmaSwitch quad nozzle body with a 25 cm nozzle spacing.

Flex-fold

The standard, electro-hydraulic Flex-fold mechanism of the boom with ContourControl controls each pivot point on the boom separately. This allows the boom to be folded out to a reduced working width from the tractor cab. The overload protection within the hydraulic system is simultaneously used as break back protection when working at these reduced working widths. The individual control of the hydraulic cylinders at the pivot points allows up to two segments to be folded out at a time on each side of the boom. This considerably reduces the down time when folding the boom in and out.

Future-proof technology

New and innovative technologies, such as row application on row crops or spot application on weeds, are now made possible for mounted sprayers.

- Fast and precise boom guidance for maximum work rates

- Distance of less than 50 cm to the target surface in conjunction with AmaSwitch fitted with quad nozzle bodies and 25 cm nozzle spacing for reduced drift

- Flex-fold for quicker folding

- Simple folding of the boom to reduced working widths with integrated break back protection for use in different tramline systems

- Future-proof technology, optimally equipped for row and spot application

The UF 2002 mounted sprayer with ContourControl for fully automatic boom guidance

Fully automatic, active boom guidance via ContourControl

AmaSwitch with quad nozzle body and off-set kit for a perfect 25 cm nozzle spacing

Exact distance to the target surface for less drift

Illustrations, content and technical data are not binding and may differ depending on the level of equipment. Country-specific road traffic regulations apply and must be complied with, meaning that special approval may be required. The permissible axle loads and total weights of the tractor should be checked. Not all the listed combination options are possible with all tractor manufacturers.

About AMAZONE

AMAZONEN-WERKE H. DREYER SE & Co. KG is based in Hasbergen-Gaste in Germany and manufactures agricultural and groundcare machinery. The owner-managed company employs more than 2000 people at nine different production sites. The agricultural machinery range includes soil tillage implements, seed drills, fertiliser spreaders and crop protection equipment. SCHMOTZER Hacktechnik has been part of the AMAZONE Group since 2019. Based on these core competencies, AMAZONE is now the specialist for intelligent crop production in agriculture. Further information: www.amazone.de